Metrix AX3220

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix AX3220.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Error codes and warnings are returned as the return value of each instrument

driver function. A program should examine this value after each call to an

instrument driver function to determine if any error occurred.  The general

meaning of the return value is as follows:

Value                   Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

Possible error codes and their meanings are listed with the corresponding

instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Metrix AX3220

instrument. The functions are listed in alphabetical order with a

description of the function, C syntax of the function, a description of each

parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Metrix AX3220                          Function Name:

Initialise                                  AX3220H_init

Configure

Set Mode                                 AX3220H_set_mode

Set Output                               AX3220H_set_op

AX3220H_set_volts

AX3220H_set_current

AX3220H_set_ovp

AX3220H_set_op_status

AX3220H_set_damping

Master Output Control                    AX3220H_master_op

Status Reporting                         AX3220H_set_status

Measurement and Status

Read Output                              AX3220H_read_op

AX3220H_read_volts

AX3220H_read_current

Read Status                              AX3220H_read_status

Local Storage

Store                                    AX3220H_store

Recall                                   AX3220H_recall

Miscellaneous

Send Command String                      AX3220H_send_cmd

Get Response String                      AX3220H_get_response

Send Block From File                     AX3220H_send_from_file

Get Block to File                        AX3220H_get_to_file

Close                                       AX3220H_close

________________________________________________________________________________

Metrix AX3220

AX3220H series power supplies

This instrument module provides GPIB support for

the AX3220H series power supplies. Instruments in

this range meet the requirements of IEEE 488.2

standard. The module will be updated from time to

time as new instruments become available in the

range.

Instruments supported are:

AX3220H         Dual 32 volt, 2 amp, Quad mode

The module contains the following functions:

1. Initialise.

2. Configuration

1. Set Mode

2. Set Output

3. Master Output control

4. Status Reporting

3. Measurement and Status.

1. Read Output

2. Read Status

4.Local Storage.

1. Store

2. Recall

5. Close

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

AX3220H_close

void AX3220H_close (void);

Purpose

Panel name:     Close

Description:    Takes the instrument off-line.

The  instrument must  be

reinitialized to use it again.

________________________________________________________________________________

AX3220H_get_response

int AX3220H_get_response (char responseString[]);

Purpose

Panel name:     Get response string

Description:    Reads a response from the AX3220x

into a string variable.

Parameter List

responseString

Variable Type       char []

Control name:   Response Stirng

Description:    This control specifies a string

variable to receive the response

from the AX3220x. The string must

be long enough to receive the

response to avoid overflow. To aid

in checking this the number of

bytes put into the string is

returned from the function. The

absolute maximum length allowed is

2000 bytes.

Variable type:  String

Return Value

Control name:   Number of Bytes

Description:    This value is the number of bytes

returned by the AX3220x and

placed in the string.

Variable type:  Integer

________________________________________________________________________________

AX3220H_get_to_file

void AX3220H_get_to_file (char filename[]);

Purpose

Panel name:     Get block to file

Description:    Gets a binary block from the

AX2220x and writes it to a disc

file.

Note            This operation is usefull for

Learn and Store blocks.

Parameter List

filename

Variable Type       char []

Control name:   File name

Description:    This control specifies file name

to which the binary block will be

written when read from the

AX3220x.

Variable type:  String - max length 50 characters

________________________________________________________________________________

AX3220H_init

void AX3220H_init (int GPIBAddress, int setDefaults, int performSelfTest);

Purpose

Panel name:     Initialise

Description:

Initialises the AX3220x in the following way

- opens the instrument and sets the GPIB

address to the specified value.

- sends device clear and clear status

commands.

- queries the instrument to determine the

type/model number.

- optionally sets the instrument to its

default state.

- optionally performs a self test on the

instrument.

Parameter List

GPIBAddress

Variable Type       int

Control name:   GPIB Address

Description:    This control specifies the

address used to communicate with

the AX3220x power supply. It must

correspond with the value set up

at the instrument front panel.

Variable type:  Integer

Valid range:    0 to 30

Default value:  11

setDefaults

Variable Type       int

Control name:   Set Defaults

Description:    When set to Yes the parameters

in the instrument will be set to

their default values. If set to No

the instrument settings will not

be changed.

Variable type:  Integer

Valid range:    No = 0 , Yes = Any non-zero value

Default value:  Yes

performSelfTest

Variable Type       int

Control name:   Perform Self Test

Description:    Specifies if a self test should

be performed or not. If the test

is performed the returned value

will be in the global variable

AX3220H_err. A value of zero

indicates that the test passed.

See the Error control for a list

of possible returned values.

Variable type:  Integer

Valid range:    No = 0 , Yes = Any non-zero value

Default value:  No

________________________________________________________________________________

AX3220H_master_op

void AX3220H_master_op (int masterOutput);

Purpose

Panel name:     Master Output Control

Description:

Set master output switch on or off.

Parameter List

masterOutput

Variable Type       int

Control name:   Master Output

Description:    Used to set the master output

switch On or Off. When On The

voltage on all outputs, which

have their individual Output

Status switches set to On, will

appear at the output terminals of

the instrument.

Variable type:  integer

Valid range:    Off = 0 , On = any non-zero value

default value:  Off

________________________________________________________________________________

AX3220H_read_current

void AX3220H_read_current (int outputNumber, double *outputCurrent);

Purpose

Panel name:     Read Output

Description:

Returns the voltage and/or current from the

selected output.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function read_volts returns the

output voltage on the selected output.

The secondary function read_current returns the

current being drawn from the selected output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number from

which the values will be read.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

outputCurrent

Variable Type       double (passed by reference)

Control Name:   Output current

Description:    Current being output by the power

supply.

Control Type:   Real

________________________________________________________________________________

AX3220H_read_op

void AX3220H_read_op (int outputNumber);

Purpose

Panel name:     Read Output

Description:

Returns the voltage and/or current from the

selected output.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function read_volts returns the

output voltage on the selected output.

The secondary function read_current returns the

current being drawn from the selected output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number from

which the values will be read.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

________________________________________________________________________________

AX3220H_read_status

int AX3220H_read_status (int readStatus);

Purpose

Panel name:     Read Status

Description:    Returns the byte value from the

selected status register.

Parameter List

readStatus

Variable Type       int

Control name:   Read Status

Description:    Used to select which status

register to read

Variable type:  integer

Valid range:    min = 0 , max = 6

Default value   0 (Status Byte)

Return Value

Control Name:   Value

Description:    Retuened byte value of the

selected register.

Control Type:   integer

Valid range:    0 to 255 - register value

-1 - an error was encountered.

________________________________________________________________________________

AX3220H_read_volts

void AX3220H_read_volts (int outputNumber, double *outputVoltage);

Purpose

Panel name:     Read Output

Description:

Returns the voltage and/or current from the

selected output.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function read_volts returns the

output voltage on the selected output.

The secondary function read_current returns the

current being drawn from the selected output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number from

which the values will be read.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

outputVoltage

Variable Type       double (passed by reference)

Control Name:   Output voltage

Description:    Voltage being output from the

power supply.

Control Type:   Real.

________________________________________________________________________________

AX3220H_recall

void AX3220H_recall (int storenumber);

Purpose

Panel name:     Recall

Description:    Recalls an instrument set up from

a previous store operation.

Parameter List

storenumber

Variable Type       int

Control name:   Store number

Description:    Select the number of the store

from which to recall the

instrument set up.

Variable type:  integer

Valid range:    1 to 25

________________________________________________________________________________

AX3220H_send_cmd

void AX3220H_send_cmd (char commandString[]);

Purpose

Panel name:     Send command string

Description:    Sends a string of bytes to the

AX3220x from a string variable.

Parameter List

commandString

Variable Type       char []

Control name:   Command String

Description:    This control specifies data to be

sent to the AX3220x. The absolute

maximum length allowed is 2000

bytes.

Variable type:  String

________________________________________________________________________________

AX3220H_send_from_file

void AX3220H_send_from_file (char filename[]);

Purpose

Panel name:     Send block from file

Description:    Sends a binary block to the

AX3220x from a disc file.

Note            This operation is usefull for

Learn and Store blocks.

Parameter List

filename

Variable Type       char []

Control name:   File name

Description:    This control specifies file name

from which the binary block will

be sent to the AX3220x.

Variable type:  String - max length 50 characters

________________________________________________________________________________

AX3220H_set_current

void AX3220H_set_current (int outputNumber, double current);

Purpose

Panel name:     Set Output

Description:

Set voltage, current, OVP, output status and

readback meter damping for each output. The

values which are legal depend on the instrument

type and the Mode and Tracking specification.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function set_volts sets the voltage

on the selected output.

The secondary function set_current sets the

current limit on the selected output.

The secondary function set_ovp sets the over

voltage limit on the selected output.

The secondary function, set output status,

specifies whether the output status of the

specified output is on or off.

The secondary function, set damping, specifies

the reading meter damping for the voltage and

current of the specified output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

current

Variable Type       double

Control name:   Current

Description:    The current limit value to which

the specified output will be set.

The value is expressed in Amps.

Variable type:  real

Valid range:    instrument dependent

________________________________________________________________________________

AX3220H_set_damping

void AX3220H_set_damping (int outputNumber, int meterDamping);

Purpose

Panel name:     Set Output

Description:

Set voltage, current, OVP, output status and

readback meter damping for each output. The

values which are legal depend on the instrument

type and the Mode and Tracking specification.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function set_volts sets the voltage

on the selected output.

The secondary function set_current sets the

current limit on the selected output.

The secondary function set_ovp sets the over

voltage limit on the selected output.

The secondary function, set output status,

specifies whether the output status of the

specified output is on or off.

The secondary function, set damping, specifies

the reading meter damping for the voltage and

current of the specified output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

meterDamping

Variable Type       int

Control name:   Meter Damping

Description:    Specifies the dampimg applied to

the readback metering of the

selected output.

Variable type:  integer

Valid range:    On = 0 , Off = any non-zero

value

default value:  Off

________________________________________________________________________________

AX3220H_set_mode

void AX3220H_set_mode (int mode, int trackingA, int trackingB, int tripMode);

Purpose

Panel name:     Set Mode

Description:

Set the instrument to the required mode. Modes

available are instrument dependant. Tracking is

possible only between outputs which are

internally isolated. The number of legal modes is

instrument dependent.

AX3220H Modes:

Mode No.            Description

------------   ------------------------------

0          Outputs 1 and 2 isolated.

Outputs 1 and 2 may be set to

tracking (Tracking A = 3).

1         Outputs 1 and 2 series connected.

Tracking not possible.

2         Outputs 1 and 2 parallel

connected.

Tracking not possible.

Parameter List

mode

Variable Type       int

Control name:   Mode

Description:    Selects The required mode.

Variable type:  Integer

Valid range:    Instrument dependent

(maximum is 0 to 13)

Default value:  0 (all outputs isolated)

trackingA

Variable Type       int

Control name:   Tracking A

Description:    Selects one of two possible

tracking specifications.

Variable type:  Integer

Valid range:    Instrument dependent - see manual

Default value:  0 - no tracking

trackingB

Variable Type       int

Control name:   Tracking B

Description:    Selects one of two possible

tracking specifications. This

value should be set to zero for

all except quadruple output

instruments.

Variable type:  Integer

Valid range:    Instrument dependent - see manual

Default value:  0 - no tracking

tripMode

Variable Type       int

Control name:   Trip Mode

Description:    Specifies the action to be taken

when any output trips for any

reason - ovp or temperature. When

set to All then the complete

instrument is shut down - the

Master On/Off switch is set to

Off. When set to Single then only

the output which tripped is shut

down - Output Status is set Off.

Variable type:  Integer

Valid range:    All = 0 , Single = any non-zero

value.

Default value:  All

________________________________________________________________________________

AX3220H_set_op

void AX3220H_set_op (int outputNumber);

Purpose

Panel name:     Set Output

Description:

Set voltage, current, OVP, output status and

readback meter damping for each output. The

values which are legal depend on the instrument

type and the Mode and Tracking specification.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function set_volts sets the voltage

on the selected output.

The secondary function set_current sets the

current limit on the selected output.

The secondary function set_ovp sets the over

voltage limit on the selected output.

The secondary function, set output status,

specifies whether the output status of the

specified output is on or off.

The secondary function, set damping, specifies

the reading meter damping for the voltage and

current of the specified output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

________________________________________________________________________________

AX3220H_set_op_status

void AX3220H_set_op_status (int outputNumber, int outputStatus);

Purpose

Panel name:     Set Output

Description:

Set voltage, current, OVP, output status and

readback meter damping for each output. The

values which are legal depend on the instrument

type and the Mode and Tracking specification.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function set_volts sets the voltage

on the selected output.

The secondary function set_current sets the

current limit on the selected output.

The secondary function set_ovp sets the over

voltage limit on the selected output.

The secondary function, set output status,

specifies whether the output status of the

specified output is on or off.

The secondary function, set damping, specifies

the reading meter damping for the voltage and

current of the specified output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

outputStatus

Variable Type       int

Control name:   Output Status

Description:    Specifies the status of the

specified output.

Variable type:  integer

Valid range:    Off = 0 , On = any non-zero value

default value:  Off

________________________________________________________________________________

AX3220H_set_ovp

void AX3220H_set_ovp (int outputNumber, double OVP);

Purpose

Panel name:     Set Output

Description:

Set voltage, current, OVP, output status and

readback meter damping for each output. The

values which are legal depend on the instrument

type and the Mode and Tracking specification.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function set_volts sets the voltage

on the selected output.

The secondary function set_current sets the

current limit on the selected output.

The secondary function set_ovp sets the over

voltage limit on the selected output.

The secondary function, set output status,

specifies whether the output status of the

specified output is on or off.

The secondary function, set damping, specifies

the reading meter damping for the voltage and

current of the specified output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

OVP

Variable Type       double

Control name:   OVP

Description:    The over voltage protection value

to which the specified output will

be set. The value is expressed in

Volts.

Variable type:  real

Valid range:    instrument dependent

________________________________________________________________________________

AX3220H_set_status

void AX3220H_set_status (int setStatusEnable, int value);

Purpose

Panel name:     Status Reporting

Description:

Specify the way in which status is reported and

Service Request is generated as defined in the

IEEE 488.2 standard.

The function provides access to the four status

enable registers in the AX3220x power supply,

these are:

Service Request Enable Register

- Specifies which bits, when set, in the

Status Byte register will generate a

Service Request.

Standard Event Enable Register

- Specifies which bits, when set, in the

Standard Event Status register will set

the ESB bit in the Status Byte Register.

Limit Event Register

- Specifies which bits, when set, in the

Limit Event Status register will set the

LIM bit in the Status Byte Register.

Parallel Poll Enable Register

- Specifies which bits, when set, in the

Status Byte register will be used to form

the ist local message

See the IEEE 488.2 Standard and the instrument

instruction book for a full description of status

reporting capabilities.

Parameter List

setStatusEnable

Variable Type       int

Control name:   Set Status Enable

Description:    Used to select which enable

register will receive the byte

value.

Variable type:  integer

Valid range:    min = 0 , max = 3

Default value   0 (SRQ Enable)

value

Variable Type       int

Control name:   Value

Description:    A byte value which will be written

to the selected enable register.

Variable type:  integer

Valid range:    0 to 255

Default value:  0

________________________________________________________________________________

AX3220H_set_volts

void AX3220H_set_volts (int outputNumber, double voltage);

Purpose

Panel name:     Set Output

Description:

Set voltage, current, OVP, output status and

readback meter damping for each output. The

values which are legal depend on the instrument

type and the Mode and Tracking specification.

The output number is a common control which is

used for all other functions which are secondary.

The primary function set_op is a dummy and does

not execute any code.

The secondary function set_volts sets the voltage

on the selected output.

The secondary function set_current sets the

current limit on the selected output.

The secondary function set_ovp sets the over

voltage limit on the selected output.

The secondary function, set output status,

specifies whether the output status of the

specified output is on or off.

The secondary function, set damping, specifies

the reading meter damping for the voltage and

current of the specified output.

Parameter List

outputNumber

Variable Type       int

Control name:   Output Number

Description:    Indicates the output number to

which the commands will be sent.

Variable type:  integer

Valid range:    min = 1 , max = number of outputs

available for the particular

instrument.

Default value   1

voltage

Variable Type       double

Control name:   Voltage

Description:    The voltage value to which the

specified output will be set. The

value is expressed in Volts.

Variable type:  real

Valid range:    instrument dependent

________________________________________________________________________________

AX3220H_store

void AX3220H_store (int storenumber);

Purpose

Panel name:     Store

Description:    Saves the complete state of the

AX3220x to local non-volatile

memory.

Parameter List

storenumber

Variable Type       int

Control name:   Store number

Description:    Select the number of the store to

receive the instrument set up.

Variable type:  integer

Valid range:    1 to 25