Metrix MTX3240 Function Generator

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix MTX3240 Function Generator.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Metrix MTX3240 Function Generator

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                    Function Name:

Initialize                        mtx3240_init

Configure                         mtx3240_config

Sweep                             mtx3240_sweep

Frequency count                   mtx3240_measure

Close                             mtx3240_close

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

mtx3240_close

int mtx3240_close (void);

Purpose

Panel Name:    Close

Description:   Closes the instrument and takes it

off line.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-24   Invalid parity mode

-34   Illegal number of data bits

-44   Illegal number of stop bits

-90   Bad file handle

-91   Error in performing file I/O

-94   Invalid count - must be 0 or greater

-97   Invalid interrupt level

-98   Default port address not defined for ports 9 through 32

-99   I/O operation timed out

-104   Break time must be between 0 and 255

-114   Invalid input queue size

-124   Invalid output queue size

-151   General I/O Error

-152   Buffer parameter is NULL

-257   No acknowledgment received after packet sent

-258   Packet not sent within retry limit

-259   Packet not received within retry limit

-260   Unexpected end of transmission received

-261   Unable to read packet number

-262   Packet number inconsistency

-263   Packet data could not be read

-264   Checksum could not be read

-265   Received checksum does not match computed checksum

-269   Invalid packet size

-300   Error opening file

-301   Error reading file

-302   Initial negative acknowledgment not received

-303   Acknowledgment not received following end of

transmission

-304   Error while writing to file

-305   Did not receive start of data nor end of

transmission when one was expected

-402   Transfer was cancelled because CAN byte was received

-503   Invalid start delay

-504   Invalid maximum number of tries

-505   Invalid wait period

-506   Invalid packet size

-507   Unable to read CRC

-508   CRC error

________________________________________________________________________________

mtx3240_config

int mtx3240_config (int function, double frequency_Hz,

double amplitude_Vpp, double offset_V, int duty_,

double logicHigh_V, double logicLow_V);

Purpose

Panel Name:    Config

Description:   Sets up Function, Frequency,

Peak-Peak Output level, DC Offset,

Symmetry and Logic levels.

Parameter List

function

Variable Type       int

Control Name:  Function

Description:   Specifies the output waveform

function.

Variable Type: Integer

Valid Range:   0 = Sine

1 = Triangle

2 = Square

3 = Logic

4 = DC

5 = Frequency count

Default Value: 0 - Sine

frequency_Hz

Variable Type       double

Control Name:  Frequency

Description:   The output Frequency expressed in

Hz.

Variable Type: double

Valid Range:   0.1 to 5.1e+6 (0.1Hz to 5.1MHz)

Default Value: 10kHz

amplitude_Vpp

Variable Type       double

Control Name:  Pk-Pk Output

Description:   Specifies the output level as a

peak to peak EMF value expressed

in Volts.

Variable Type: double

Valid Range:   000mV to 20.0V.

Default Value: 20V

offset_V

Variable Type       double

Control Name:  DC Offset

Description:   Sets the output Offset voltage as

an EMF value expressed in Volts

Variable Type: double

Valid Range:   -10 to +10

Default Value: 0.0

duty_

Variable Type       int

Control Name:  Symmetry

Description:   Sets the output Symmetry as a

percentage.

Variable Type: double

Valid Range:   20% to 80%

Default Value: 50.0%

logicHigh_V

Variable Type       double

Control Name:  Logic High

Description:   Sets the output Logic High voltage

Variable Type: double

Valid Range:   -10 to +10

Default Value: 0.0

logicLow_V

Variable Type       double

Control Name:  Logic low

Description:   Sets the output Logic Low voltage

Variable Type: double

Valid Range:   -10 to +10

Default Value: 0.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-24   Invalid parity mode

-34   Illegal number of data bits

-44   Illegal number of stop bits

-90   Bad file handle

-91   Error in performing file I/O

-94   Invalid count - must be 0 or greater

-97   Invalid interrupt level

-98   Default port address not defined for ports 9 through 32

-99   I/O operation timed out

-104   Break time must be between 0 and 255

-114   Invalid input queue size

-124   Invalid output queue size

-151   General I/O Error

-152   Buffer parameter is NULL

-257   No acknowledgment received after packet sent

-258   Packet not sent within retry limit

-259   Packet not received within retry limit

-260   Unexpected end of transmission received

-261   Unable to read packet number

-262   Packet number inconsistency

-263   Packet data could not be read

-264   Checksum could not be read

-265   Received checksum does not match computed checksum

-269   Invalid packet size

-300   Error opening file

-301   Error reading file

-302   Initial negative acknowledgment not received

-303   Acknowledgment not received following end of

transmission

-304   Error while writing to file

-305   Did not receive start of data nor end of

transmission when one was expected

-402   Transfer was cancelled because CAN byte was received

-503   Invalid start delay

-504   Invalid maximum number of tries

-505   Invalid wait period

-506   Invalid packet size

-507   Unable to read CRC

-508   CRC error

________________________________________________________________________________

mtx3240_init

int mtx3240_init (int serial_port, int IDQuery, int resetDevice);

Purpose

Panel Name:    Initialize

Description:   Initializes the instrument in the

following ways:

- opens the instrument

- optionally performs an

identification query on the

instrument

- optionally sets the instrument

to a default configuration

Parameter List

serial_port

Variable Type       int

Control Name:  Serial Port

Description:   Specifies the serial port to be used to

communicate with the instrument.

Variable Type: Integer

Valid Range:   1 to 4

Default Value: 1

IDQuery

Variable Type       int

Control Name:  ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

resetDevice

Variable Type       int

Control Name:  Reset

Description:   Specifies if the instrument is to

be reset during the initialization

procedure.

Variable Type: Integer

Valid Range:   0 = No Reset

1 = Reset device

Default Value: 1 = Reset

NOTE: If you do not want the instrument reset.

Set this control to No Reset while

initializing the instrument.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-24   Invalid parity mode

-34   Illegal number of data bits

-44   Illegal number of stop bits

-90   Bad file handle

-91   Error in performing file I/O

-94   Invalid count - must be 0 or greater

-97   Invalid interrupt level

-98   Default port address not defined for ports 9 through 32

-99   I/O operation timed out

-104   Break time must be between 0 and 255

-114   Invalid input queue size

-124   Invalid output queue size

-151   General I/O Error

-152   Buffer parameter is NULL

-257   No acknowledgment received after packet sent

-258   Packet not sent within retry limit

-259   Packet not received within retry limit

-260   Unexpected end of transmission received

-261   Unable to read packet number

-262   Packet number inconsistency

-263   Packet data could not be read

-264   Checksum could not be read

-265   Received checksum does not match computed checksum

-269   Invalid packet size

-300   Error opening file

-301   Error reading file

-302   Initial negative acknowledgment not received

-303   Acknowledgment not received following end of

transmission

-304   Error while writing to file

-305   Did not receive start of data nor end of

transmission when one was expected

-402   Transfer was cancelled because CAN byte was received

-503   Invalid start delay

-504   Invalid maximum number of tries

-505   Invalid wait period

-506   Invalid packet size

-507   Unable to read CRC

-508   CRC error

________________________________________________________________________________

mtx3240_measure

int mtx3240_measure (double *frequency_Hz);

Purpose

Panel Name:    Measure frequency

Description:   returns the measured frequency.

Parameter List

frequency_Hz

Variable Type       double (passed by reference)

Control Name:  Frequency

Description:   The input Frequency expressed in Hz.

Variable Type: double

Valid Range:   5.0 to 100e+6 (5Hz to 100MHz)

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-24   Invalid parity mode

-34   Illegal number of data bits

-44   Illegal number of stop bits

-90   Bad file handle

-91   Error in performing file I/O

-94   Invalid count - must be 0 or greater

-97   Invalid interrupt level

-98   Default port address not defined for ports 9 through 32

-99   I/O operation timed out

-104   Break time must be between 0 and 255

-114   Invalid input queue size

-124   Invalid output queue size

-151   General I/O Error

-152   Buffer parameter is NULL

-257   No acknowledgment received after packet sent

-258   Packet not sent within retry limit

-259   Packet not received within retry limit

-260   Unexpected end of transmission received

-261   Unable to read packet number

-262   Packet number inconsistency

-263   Packet data could not be read

-264   Checksum could not be read

-265   Received checksum does not match computed checksum

-269   Invalid packet size

-300   Error opening file

-301   Error reading file

-302   Initial negative acknowledgment not received

-303   Acknowledgment not received following end of

transmission

-304   Error while writing to file

-305   Did not receive start of data nor end of

transmission when one was expected

-402   Transfer was cancelled because CAN byte was received

-503   Invalid start delay

-504   Invalid maximum number of tries

-505   Invalid wait period

-506   Invalid packet size

-507   Unable to read CRC

-508   CRC error

________________________________________________________________________________

mtx3240_sweep

int mtx3240_sweep (int enable, double start_frequency_Hz,

double stop_frequency_Hz, double time_sec, int mode,

int source);

Purpose

Panel Name:    Sweep

Description:   Enables Sweep (Software Controlled

Frequency) and sets up the Mode,

Start and Stop Frequencies and the

sweep time.

Parameter List

enable

Variable Type       int

Control Name:  Enable

Description:   Sets the status of Sweep.

Variable Type: Integer

Valid Range:

0 = Off

1 = On

Default Value: 0 - Off

start_frequency_Hz

Variable Type       double

Control Name:  Start Frequency

Description:   The Sweep Start Frequency

expressed in Hz.

Variable Type: double

Valid Range:   0.1 to 5.1e+6 (0.1Hz to 5.1MHz)

Default Value: none

stop_frequency_Hz

Variable Type       double

Control Name:  Stop Frequency

Description:   The Sweep Stop Frequency

expressed in Hz.

Variable Type: double

Valid Range:   0.1 to 5.1e+6 (0.1Hz to 5.1MHz)

Default Value: none

time_sec

Variable Type       double

Control Name:  Time

Description:   Specifies the internal Sweep ramp

time in seconds.

Variable Type: double

Valid Range:   10e-2 to 10.000

Default Value: none

mode

Variable Type       int

Control Name:  Mode

Description:   Sets the Sweep Mode.

Variable Type: Integer

Valid Range:   0 = Linear

1 = Logarithmic

Default Value: 0 - linear

source

Variable Type       int

Control Name:  Source

Description:   Sets the Sweep Source.

Variable Type: Integer

Valid Range:   0 = Internal

1 = External

Default Value: 0 - Internal

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-24   Invalid parity mode

-34   Illegal number of data bits

-44   Illegal number of stop bits

-90   Bad file handle

-91   Error in performing file I/O

-94   Invalid count - must be 0 or greater

-97   Invalid interrupt level

-98   Default port address not defined for ports 9 through 32

-99   I/O operation timed out

-104   Break time must be between 0 and 255

-114   Invalid input queue size

-124   Invalid output queue size

-151   General I/O Error

-152   Buffer parameter is NULL

-257   No acknowledgment received after packet sent

-258   Packet not sent within retry limit

-259   Packet not received within retry limit

-260   Unexpected end of transmission received

-261   Unable to read packet number

-262   Packet number inconsistency

-263   Packet data could not be read

-264   Checksum could not be read

-265   Received checksum does not match computed checksum

-269   Invalid packet size

-300   Error opening file

-301   Error reading file

-302   Initial negative acknowledgment not received

-303   Acknowledgment not received following end of

transmission

-304   Error while writing to file

-305   Did not receive start of data nor end of

transmission when one was expected

-402   Transfer was cancelled because CAN byte was received

-503   Invalid start delay

-504   Invalid maximum number of tries

-505   Invalid wait period

-506   Invalid packet size

-507   Unable to read CRC

-508   CRC error